KrishiAI

Complete Hardware + IoT + Edge AI Ecosystem

Startup-Grade Architecture Document — Version 1.0 | May 2026 | CONFIDENTIAL

AI Smart Farmer Decision Platform for Rural India

Low Cost  |  High Reliability  |  Offline-First  |  Scalable to 1 Lakh+ Farmers

# Section 0: KrishiAI Software Architecture Analysis

A deep analysis of the existing KrishiAI codebase was performed before designing any hardware. This ensures every hardware component maps directly to a real API endpoint, database model, or workflow already present in the software.

## 0.1  Tech Stack Summary

| Layer | Technology | Status |
| --- | --- | --- |
| Backend Framework | FastAPI (Python 3.11+) + Uvicorn | Production-ready |
| Database | SQLAlchemy ORM — SQLite (dev) / PostgreSQL | Production-ready |
| Cache | Redis (in-memory dict fallback) | Production-ready |
| LLM / AI | OpenAI GPT-4o-mini / Google Gemini 2.0 Flash (auto-routing) | Production-ready |
| STT | OpenAI Whisper-1 + Browser Web Speech API | Production-ready |
| TTS | Sarvam Bulbul → Google Neural2 → OpenAI TTS-1 → Browser fallback | Production-ready |
| Crop ML Model | scikit-learn Random Forest (.pkl slot ready) | Mock — needs real training |
| Disease Detection | CNN on PlantVillage — endpoint scaffolded at POST /api/v1/disease/detect | Planned |
| Weather Providers | OWM → WeatherAPI → Tomorrow.io → AccuWeather (auto-fallback) | Production-ready |
| Mandi Prices | Static mock DB; slot ready for e-NAM / Agmarknet integration | Mock — needs live feed |
| Worker Connect | Full CRUD REST API + worker search + wage hints | Production-ready |
| Frontend | Next.js 14 PWA (TypeScript) with Tailwind CSS | Production-ready |
| Rate Limiting | slowapi per-IP (60 req/min weather, 30 chat) | Production-ready |

## 0.2  Existing API Endpoints — Hardware Integration Points

| Endpoint | Method | Hardware Integration Required |
| --- | --- | --- |
| POST /api/ml/recommend | POST | ESP32 sensor node reads N, P, K, pH, temp, humidity, rainfall and POSTs here for crop recommendation |
| POST /api/v1/disease/detect | POST | ESP32-CAM or RPi Camera captures leaf JPEG and POSTs as multipart/form-data |
| GET /api/weather/current | GET | RPi Gateway fetches weather using its GPS coordinates; local sensors (DHT22+BMP280) supplement/cross-validate |
| POST /api/chat/message (SSE) | POST | Farmer voice query → mic captures audio → Whisper STT → chatbot → TTS speaker output |
| POST /api/chat/stt | POST | Raw microphone audio uploaded from field device for transcription |
| POST /api/chat/tts | POST | AI reply text converted to audio for speaker playback on village kiosk |
| GET /api/mandi/crops | GET | Gateway caches prices every 6h for offline display on kiosk LCD |
| GET /api/schemes | GET | Gateway caches scheme info for offline farmer access |
| POST /api/worker-connect/jobs | POST | SMS-triggered job posting from SIM800L via webhook to chatbot |

## 0.3  Database Models Relevant to Hardware

| DB Model | Key Fields | Hardware Data Source |
| --- | --- | --- |
| User | phone_number, lat, lon, language | GPS module for location; phone number = device auth key |
| FarmerField | polygon_geojson, soil_type, crop_id, sowing_date | GPS boundary-walk by farmer; soil_type derived from NPK+pH sensor readings |
| DiseaseDetection | image_url, detected_disease, confidence, remedy_suggested | Camera capture from ESP32-CAM or RPi Camera Module v2 |
| ChatInteraction | query_text, query_audio_url, response_text, language | Microphone input; speaker output on village kiosk device |
| WorkerJob | work_type, location, workers_needed, wage, contact_number | Farmer posts via SMS (SIM800L webhook) or PWA app |

## 0.4  New API Endpoints Required to Activate Hardware

| New Endpoint (to add) | Purpose | Payload |
| --- | --- | --- |
| POST /api/sensors/batch | Batch upload sensor readings from gateway | {farm_id, readings:[{N,P,K,pH,moisture,temp,rain,ts}]} |
| POST /api/v1/disease/detect | CNN disease detection (scaffolded, needs model) | multipart/form-data: image file + {crop_type, farm_id} |
| POST /api/devices/register | Register new IoT node with backend | {phone_number, device_type, mac_address, firmware_version} |
| GET /api/irrigation/schedule | Fetch AI-generated irrigation schedule for farm | Query: ?farm_id=&days=7 |
| POST /api/irrigation/override | Manual pump/valve command from farmer app | {farm_id, zone_id, action:on/off, duration_min} |
| GET /api/ota/firmware/{type} | Check and fetch OTA firmware update | Query: ?current_version=1.2.3 → {url, sha256, version} |
| POST /api/alerts/sms-webhook | Receive SMS from SIM800L / Twilio for chatbot | {from_number, message_body} → {reply_message} |

KEY INSIGHT FROM SOFTWARE ANALYSIS: The KrishiAI FastAPI backend accepts N, P, K, temperature, humidity, pH, rainfall as POST body fields. This is EXACTLY the data an ESP32 sensor node with NPK + pH + DHT22 + rain gauge can supply. Hardware must be designed to produce these 7 values reliably — they are the core input to the AI crop engine. The disease detection endpoint accepts a JPEG image upload — ESP32-CAM outputs JPEG natively. The chatbot uses SSE streaming — the RPi Gateway must buffer and relay these events to the farmer speaker. The mandi and scheme APIs are read-only — Gateway caches them offline, no sensor hardware needed.

# Section 1: Complete Hardware Architecture

## 1.1  Three-Tier System Architecture

KrishiAI hardware uses a three-tier architecture: Field Sensor Nodes (Tier 1), Village AI Gateway (Tier 2), and Cloud Backend (Tier 3). Each tier communicates upward and the system is designed for graceful degradation — Tier 1 keeps irrigating even if both Tier 2 and Tier 3 are offline.

KRISHIAI HARDWARE ARCHITECTURE DIAGRAM ┌─────────────────────────────────────────────────────────────────┐ │ TIER 3 — CLOUD │ │ FastAPI Backend (Render / AWS Mumbai) + PostgreSQL + Redis │ │ OpenAI / Gemini LLM ¦ Weather APIs ¦ OTA Update Server │ └─────────────────────┬───────────────────────────────────────────┘ │ HTTPS REST + MQTT over TLS (4G / WiFi) ┌─────────────────────▼───────────────────────────────────────────┐ │ TIER 2 — VILLAGE AI GATEWAY │ │ Raspberry Pi 4 (2 GB) + 4G LTE USB Dongle (SIM7600) │ │ TFLite Disease Model ¦ Mosquitto MQTT ¦ SQLite cache │ │ LoRa SX1278 RX ¦ SIM800L SMS ¦ WiFi AP ¦ Speaker+Mic │ │ RPi Camera v2 ¦ 3.5" TFT LCD dashboard │ └─────────────────────┬───────────────────────────────────────────┘ │ LoRa 433 MHz / ESP-NOW / WiFi 2.4 GHz ┌─────────────────────▼───────────────────────────────────────────┐ │ TIER 1 — FIELD SENSOR NODES (1 per 2 acres) │ │ ESP32 DevKit v1 + NPK RS485 + pH + Capacitive Moisture │ │ DHT22 + BME280 + BH1750 + Tipping-Bucket Rain Gauge │ │ ESP32-CAM (OV2640) + SX1278 LoRa + 4-ch Relay │ │ 12V Latching Solenoids + 12V 7Ah SLA + 20W Solar Panel │ │ IP66 enclosure on 3 m GI pole │ └─────────────────────────────────────────────────────────────────┘

## 1.2  Hardware Component Selection Rationale

| Component | Selected Model | Why Selected | INR Cost | Alternative |
| --- | --- | --- | --- | --- |
| Main MCU (Field Node) | ESP32 DevKit v1 (38-pin WROOM-32) | Dual-core 240 MHz, WiFi+BT built-in, 36 GPIO, ADC, I2C, SPI, UART, 4 MB Flash, 520 KB SRAM. Best value MCU for IoT in India. | 250–350 | ESP8266 (no BLE, fewer GPIO) |
| AI Gateway SBC | Raspberry Pi 4 Model B 2 GB | Quad-core ARM Cortex-A72 @ 1.8 GHz, runs TFLite Python, Linux, dual-band WiFi, USB 4G, GPIO for peripherals. | 4,500–5,500 | Orange Pi 3B (cheaper); Jetson Nano (overkill for MVP) |
| Field Camera | ESP32-CAM with OV2640 2MP | 2MP JPEG camera built into the ESP32 module. Compact, single-unit, sends JPEG over WiFi. Perfect for button-triggered leaf capture. | 280–350 | Arducam 5 MP Mini SPI (₹600, better quality) |
| Gateway Camera | Raspberry Pi Camera Module v2 (IMX219 8MP) | High-resolution for accurate TFLite inference. CSI interface = zero CPU USB overhead. Python picamera2 library. | 1,800–2,200 | Arducam USB-C camera (more universal) |
| LoRa Module | SX1278 Ra-02 433 MHz | 3–5 km open-field range, ultra-low power (<2 mA RX), SPI interface, <₹250 per module. Dominant IoT standard in rural India. | 200–280 | RFM95W (same SX127x chip, different PCB layout) |
| GSM Module | SIM800L (SMS) / SIM7600G (4G data) | SIM800L: SMS-only, ₹350, 2G fallback. SIM7600G: full 4G LTE for gateway internet, USB dongle form factor. | 350–1,800 | A9G (GPS+GSM combo for single-chip solution) |
| Soil Moisture | Capacitive Sensor v1.2 (corrosion-free) | Capacitive type measures dielectric constant — no galvanic contact so no corrosion. 3–5 year lifespan vs 1–2 months for resistive. | 80–120 | FC-28 resistive (₹30, corrodes in monsoon season) |
| Soil NPK + pH + EC | RS-ECPHTH-N01 7-in-1 RS485 MODBUS | Simultaneously measures N, P, K, EC, pH, moisture, temperature via RS485. Industrial waterproof grade. Single probe supplies all inputs to /api/ml/recommend. | 2,500–3,500 | Separate NPK electrodes (more wiring, less accurate) |
| Relay Module | 4-Channel Optocoupled 5 V | Optocoupler isolates ESP32 GPIO from pump/valve 220 V AC side. 10 A/250 VAC per channel. Most common IoT relay in India. | 120–180 | MOSFET driver board (DC loads only, no AC isolation) |
| Solenoid Valve | 12 V DC Latching 3/4-inch BSP | Latching type holds open/closed with a single 50 ms pulse — draws ZERO continuous power. Critical for solar battery life. | 350–500 | 12 V non-latching (draws 500 mA continuously when open — drains battery) |
| Solar Panel | 20 W 12 V Polycrystalline | 20 W sized 3× over theoretical minimum (7 W) to handle monsoon cloud cover, dust, bird droppings. Poly = best cost/watt in India. | 600–900 | 18 W Monocrystalline (slightly more efficient, 20% higher cost) |
| Solar Controller | PWM 10 A (field node) / MPPT 20 A (gateway) | PWM sufficient for small 20 W node. MPPT for 40 W gateway gives 15–30% more harvest on cloudy days. | 150–1,200 | TP4056 (Li-Ion only, 1 A max — not suitable here) |
| Battery (Node) | 12 V 7 Ah Sealed Lead Acid (SLA) | Handles 0–50°C without special BMS, cheap, widely available at every Indian electrical shop. 53-day autonomy at node duty cycle. | 400–600 | 18650 Li-Ion pack (lighter but needs BMS, sensitive to heat) |
| Battery (Gateway) | 12 V 18 Ah SLA | Powers RPi 4 + modem + display for 20+ hours on a cloudy day. Paired with 40 W solar = positive energy balance daily. | 800–1,200 | LiFePO4 12 V 10 Ah (4× longer cycle life, 3× cost) |
| Enclosure | IP66 Polycarbonate 150×100×70 mm | IP66 = dustproof + water jet resistant. UV-stabilised polycarbonate survives 5+ years in Indian sun. Available at electrical shops. | 150–300 | PVC electrical box (less UV-resistant, lower IP rating) |

# Section 2: Sensor System Design

## 2.1  7-in-1 Soil Sensor — RS-ECPHTH-N01 (Primary Sensor)

| Attribute | Details |
| --- | --- |
| Recommended Model | RS-ECPHTH-N01-TR — 7-in-1: N + P + K + EC + pH + Moisture + Temperature via RS485 MODBUS |
| Working Principle | Ion-selective electrodes for NPK; capacitive for moisture; platinum RTD for temperature. All in one stainless-steel probe. |
| Wiring | RS485 A/B lines → MAX485 module → ESP32 UART2 (GPIO16 RX, GPIO17 TX, GPIO12 DE/RE). Power: 9–24 V DC from 12 V rail. |
| Voltage / Current | 9–24 V DC supply, 150 mA max. Power-gate with P-MOSFET when not sampling to save battery. |
| Accuracy | N/P/K: ±2 % FS; pH: ±0.3; EC: ±2 %; Temperature: ±0.5°C; Moisture: ±3 % |
| Lifespan | 5+ years with monthly cleaning |
| Cost in India | ₹2,500–3,500 (Robu.in, IndiaMART suppliers) |
| MODBUS Registers | N=0x0000, P=0x0001, K=0x0002, Moisture=0x0012, Temperature=0x0013, EC=0x0015, pH=0x0006 |
| Calibration | Factory-calibrated. Annual recalibration with NIST standard soil samples using MODBUS register 0x06 calibration command. |
| Maintenance | Monthly: clean stainless electrodes with distilled water + soft nylon brush. Avoid prolonged contact with high-acid (pH<4) soil. |
| API Integration | N, P, K, pH, temperature, moisture values sent directly to POST /api/ml/recommend. Supplies all 7 required input fields. |

## 2.2  Capacitive Soil Moisture Sensor v1.2 (Backup / Low-Cost Option)

| Attribute | Details |
| --- | --- |
| Recommended Model | Capacitive Soil Moisture Sensor v1.2 (STEMMA QT compatible) |
| Working Principle | Measures the dielectric constant of surrounding soil via RC oscillator frequency shift. No galvanic contact = no corrosion. |
| Wiring | VCC → 3.3 V, GND → GND, AOUT → ESP32 GPIO34 (ADC1_CH6). No level shifter needed. |
| Range / Accuracy | Outputs 0–3.3 V analog; relative accuracy ±3 % after 2-point calibration in dry and saturated soil. |
| Lifespan | 3–5 years (no exposed metal). Replace if readings drift >5 % vs reference. |
| Cost | ₹80–120 (Amazon/Robu.in) |
| Calibration | Record ADC value in completely dry soil (0 %) and fully saturated (100 %). Store as NVS constants. Linear interpolation. |
| Maintenance | Wipe probe monthly with dry cloth. Quarterly: recheck calibration vs saturated/dry reference. |

## 2.3  Temperature + Humidity — DHT22 and SHT31

| Sensor | DHT22 (AM2302) | SHT31-D (Premium) |
| --- | --- | --- |
| Interface | Single-wire digital (proprietary) | I2C at 0x44 or 0x45 |
| Range | -40 to +80°C; 0–100 % RH | -40 to +125°C; 0–100 % RH |
| Accuracy | ±0.5°C; ±2 % RH | ±0.3°C; ±2 % RH |
| Max sample rate | 0.5 Hz (every 2 s) | 1 Hz |
| Wiring | GPIO with 10 kΩ pullup to 3.3 V | SDA=GPIO21, SCL=GPIO22 |
| Cost | ₹120–180 | ₹350–500 |
| Recommendation | Field nodes (budget) | Village kiosk (accuracy) |

## 2.4  Barometric Pressure — BME280

| Attribute | Details |
| --- | --- |
| Model | BME280 — pressure + temperature + humidity in one I2C/SPI chip |
| Wiring | I2C: VCC=3.3 V, SDA=GPIO21, SCL=GPIO22, SDO=GND (addr 0x76) |
| Range | 300–1100 hPa; –40 to +85°C; 0–100 % RH |
| Accuracy | ±1.0 hPa; ±0.5°C; ±3 % RH |
| Cost | ₹80–150 |
| Usage | Local pressure trend for rain prediction. Falling pressure → warn farmer of incoming storm. Also altitude correction for ET₀ formula. |

## 2.5  Tipping-Bucket Rain Gauge

| Attribute | Details |
| --- | --- |
| Model | MISOL WH-SP-RG or equivalent tipping-bucket gauge |
| Principle | Mechanical bucket tips per 0.2 mm of rainfall; each tip closes a reed switch momentarily. |
| Wiring | Reed switch between GPIO39 (input, internal pullup) and GND. Use interrupt-on-falling-edge in firmware. |
| Resolution | 0.2 mm per tip; accuracy ±5 % at 50 mm/hr |
| Cost | ₹800–1,200 |
| Firmware | Debounce: ignore triggers within 10 ms. Count tips in RTC-backed counter (survives deep sleep). Reset counter at midnight. |
| API | Accumulated mm/24 h sent as "rainfall" to POST /api/ml/recommend |

## 2.6  Water Level — HC-SR04 Ultrasonic

| Attribute | Details |
| --- | --- |
| Model | HC-SR04 ultrasonic distance sensor (2 cm–4 m, ±3 mm) |
| Wiring | VCC=5 V; TRIG=GPIO GPIO (output); ECHO=GPIO via 1 kΩ/2 kΩ voltage divider to 3.3 V; GND common. |
| Cost | ₹80–120; Float switch backup: ₹30–60 |
| Usage | Measures water depth in irrigation tank. Tank % sent via MQTT. Triggers pump stop if level < 10 %. |

## 2.7  Light / PAR Sensor — BH1750

| Attribute | Details |
| --- | --- |
| Model | BH1750 (GY-302 breakout) — 16-bit I2C ambient light, 1–65535 lux |
| Wiring | VCC=3.3 V, SDA=GPIO21, SCL=GPIO22, ADDR=GND → address 0x23 |
| Cost | ₹50–80 |
| Usage | Solar radiation proxy for ET₀ (Hargreaves method). Also detects shading events (cloud cover, crop canopy closure). |

## 2.8  Camera Sensors — Summary

| Sensor | ESP32-CAM (OV2640) | RPi Camera v2 (IMX219) | USB Webcam 1080p |
| --- | --- | --- | --- |
| Resolution | 2 MP (1600×1200 JPEG) | 8 MP (3280×2464) | 2 MP (1920×1080) |
| Edge AI | No (relays to RPi) | Yes — runs TFLite locally | No |
| Offline | No (needs WiFi/LoRa GW) | Yes (RPi TFLite) | No |
| Night vision | Add 850 nm IR LED ring | Pi NoIR removes IR filter | IR models exist |
| Cost | ₹280–350 | ₹1,800–2,200 | ₹500–1,200 |
| Best for | Field node leaf capture | Village gateway inference | Lab / office |

# Section 3: AI Camera System for Crop Disease Detection

## 3.1  Dual-Camera Strategy

A dual-camera architecture is recommended: ESP32-CAM nodes in the field for instant farmer-triggered captures, and the RPi Camera v2 at the village gateway for higher-accuracy TFLite inference. Images flow: ESP32-CAM → WiFi/LoRa → RPi Gateway (TFLite edge result in <2 s) → Cloud API (GPT-4o Vision or CNN for full diagnosis + remedy text).

## 3.2  Edge AI Inference Pipeline

EDGE AI DISEASE DETECTION FLOW STEP-BY-STEP DISEASE DETECTION PIPELINE: 1. Farmer spots disease symptom on leaf 2. Points ESP32-CAM at leaf; presses physical push button (GPIO39 wakeup) 3. ESP32-CAM wakes from deep sleep, captures JPEG at 800x600, quality 70 4. Sends JPEG via HTTP POST to RPi Gateway on local WiFi (192.168.4.x AP) [LoRa fallback]: resizes to 80x60 grayscale, sends as chunked LoRa packets 5. RPi Gateway receives image, calls tflite_inference.py: - Resize to 224x224 using OpenCV - Normalize pixels to [0, 1] - Run MobileNetV2 INT8 TFLite model (38 PlantVillage disease classes) - Returns top-3 predictions with confidence scores (<200 ms) 6. [Internet available]: RPi forwards full-res image to POST /api/v1/disease/detect Cloud model (GPT-4o Vision or trained CNN) returns detailed Hindi diagnosis + remedy 7. Result stored in disease_detections table (image_url, disease, confidence, remedy) 8. Alert pushed to farmer: - App push notification (if smartphone online) - SMS via SIM800L (if feature phone) - LCD display on village kiosk - Speaker announces diagnosis in Hindi/Kannada

## 3.3  TFLite Model Specifications

| Attribute | Details |
| --- | --- |
| Training Dataset | PlantVillage (54,306 images; 38 disease classes across 14 crops including rice, wheat, cotton, tomato, potato, maize) |
| Base Architecture | MobileNetV2 (transfer learning from ImageNet — fine-tune last 3 blocks) |
| Input Size | 224 × 224 × 3 RGB, normalized to [0.0, 1.0] |
| Output | 38-class softmax — top-3 predictions with confidence |
| Float16 Model | ~6 MB after conversion; ~280 ms inference on RPi 4 |
| INT8 Quantization | ~5 MB; ~180 ms on RPi 4; < 2 % accuracy drop vs FP32 |
| Indian Crops Added | Rice Blast, Brown Spot, BLB; Wheat Yellow/Brown Rust, Loose Smut; Cotton Bollworm, Whitefly; Tomato Late/Early Blight; Maize FAW |
| Accuracy Target | >92 % top-1 on PlantVillage test set; >80 % on India-collected validation set |
| Model OTA Update | RPi downloads new .tflite when internet restores; SHA256 verified before swap |
| Training Tool | Google Colab T4 GPU (free tier): ~2 hours training time |

## 3.4  Farmer Image Capture Methods

| Method | Hardware | Best For | Effort |
| --- | --- | --- | --- |
| Button-triggered ESP32-CAM | ESP32-CAM + push button + green LED indicator | Farmer spots specific diseased leaf; takes targeted photo | Minimal — 1 button press |
| PWA Smartphone Upload | Farmer Android phone + 4G/WiFi | Tech-savvy farmers using the KrishiAI Next.js app | Medium — open app, photo, submit |
| Auto time-lapse (fixed) | ESP32-CAM + DS3231 RTC timer wakeup at 6 AM daily | Monitoring crop canopy health trend over weeks | Zero — fully automatic |
| Fixed pole camera | RPi Camera v2 + weatherproof dome + 3 m pole | Continuous monitoring of 0.5-acre section from above | Setup only — automatic |
| Drone survey (V2) | DJI Mini 3 + KrishiAI Drone App | 50-acre aerial survey in 20 min; multispectral NDVI map | Operator-driven |

## 3.5  Night Vision and Adverse Condition Shooting

Fit ESP32-CAM enclosure with an 850 nm IR LED ring (8× IR LEDs, ₹80). The OV2640 sensor is sensitive to near-IR in Night Mode (set SPECIAL_EFFECT register). For the RPi, use the Pi NoIR Camera v2 (₹2,200 — same IMX219 without IR-cut filter). In dusty conditions a 2 mm AR-coated glass port on the enclosure protects the lens. Use automated lens cleaning (small servo + wiper brush, triggered weekly) for fixed pole cameras.

# Section 4: Communication Architecture

## 4.1  Rural India Connectivity Reality

Per TRAI 2024: ~40 % of villages have 4G but poor in-building penetration; 30 % have only 2G/3G; 10 % have no mobile coverage. WiFi is essentially absent in farm fields. KrishiAI must operate fully in all these scenarios.

## 4.2  Multi-Layer Communication Stack

| Layer | Technology | Range | Bandwidth | Power | Cost/Node | Use Case |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-Node | ESP-NOW (ESP32 peer-to-peer) | 200 m LOS | 1 Mbps | Very Low | Free (built-in) | Node-to-node mesh without a router |
| LAN | WiFi 802.11 b/g/n | 50–100 m | 150 Mbps | Medium | Free (built-in) | Node to Gateway WiFi AP (ESP32 as station) |
| Long Range | LoRa SX1278 433 MHz | 2–5 km | 0.3–50 kbps | Ultra-Low | ₹200–280 | Remote nodes beyond WiFi range to village GW |
| Wide Area | 4G LTE SIM7600G (gateway) | Cell area | 150 Mbps | High (GW only) | ₹1,800 | Gateway to cloud FastAPI backend |
| SMS Backup | GPRS SIM800L 2G | Cell area | 85 kbps | Medium | ₹350 | SMS alerts; chatbot queries on feature phones |
| BT Fallback | ESP32 BLE (Bluetooth 4.2) | 10–30 m | 1 Mbps | Low | Free (built-in) | Farmer phone direct config; debug without WiFi |
| Satellite V3 | Jio SatCom / Starlink | Nationwide | 100 Mbps+ | High | ₹3,000+/mo | Areas with zero mobile coverage |

## 4.3  Offline-First Architecture

OFFLINE OPERATION FLOW OFFLINE OPERATION GUARANTEE: The system must provide 100% irrigation automation for 7+ days with zero internet. Field Node (ESP32) offline behaviour: - Continues reading all sensors every 5 minutes — independent of connectivity - Executes stored irrigation schedule from NVS (EEPROM-equivalent) indefinitely - Buffers last 72 h of readings in SPIFFS (4 MB flash = ~5,000 readings) - Sends sensor bursts to LoRa gateway whenever link is available - Local rule: if moisture < 30% → open valve 1 for 30 min (hardcoded safety) Village Gateway (RPi) offline behaviour: - TFLite disease detection runs 100% offline — no internet needed - Chatbot uses cached SQLite knowledge base (agri facts, disease remedies) - Caches last 30 days of mandi prices, weather forecasts, scheme info - Queues all data for batch sync when internet restores - Mosquitto MQTT broker continues coordinating all field nodes Cloud sync on restore: - POST /api/sensors/batch with buffered readings (compressed JSON) - POST disease images for cloud re-analysis (may update confidence) - GET fresh AI irrigation schedule → push to nodes via MQTT - Pull OTA firmware manifest; apply updates if pending

## 4.4  MQTT Topic Architecture

| Topic Pattern | Publisher | Subscriber | Payload Summary | QoS |
| --- | --- | --- | --- | --- |
| krishiai/{farm_id}/sensors/soil | ESP32 Node | RPi GW, Cloud | JSON {N,P,K,pH,moisture,temp,EC,ts} | 1 |
| krishiai/{farm_id}/sensors/weather | ESP32 Node | RPi GW, Cloud | JSON {temp,humidity,pressure,rain_mm,lux,ts} | 1 |
| krishiai/{farm_id}/irrigation/command | RPi GW/Cloud | ESP32 Node | JSON {valve_id,state:on/off,duration_min} | 2 |
| krishiai/{farm_id}/irrigation/status | ESP32 Node | RPi GW, Cloud | JSON {valve_id,state,pump_on,flow_lpm,tank_pct} | 1 |
| krishiai/{farm_id}/camera/trigger | RPi GW/App | ESP32-CAM Node | JSON {type:leaf/auto,resolution:800x600} | 2 |
| krishiai/{farm_id}/disease/result | RPi GW | App, SMS GW | JSON {disease,confidence,remedy_hi,remedy_en} | 2 |
| krishiai/{farm_id}/alerts | RPi GW/Cloud | App, SMS GW | JSON {type,severity,message_hi,message_en,ts} | 2 |
| krishiai/gateway/{gw_id}/status | RPi GW | Cloud | JSON {online,nodes_count,internet,battery_pct} | 0 |

## 4.5  Edge-to-Cloud Data Flow

DATA FLOW: ESP32 Field Node → MQTT → RPi Gateway → FastAPI Cloud Tier 1 → Tier 2 (every 5 minutes): ESP32 reads sensors → publishes to local MQTT broker on RPi Topic: krishiai/{farm_id}/sensors/soil Tier 2 processing (RPi Gateway): 1. Stores reading in local SQLite sensor_buffer table 2. Runs local decision logic (irrigation triggers, anomaly detection) 3. If disease image received: runs TFLite → publishes result back to node Tier 2 → Tier 3 (every 15 min, or on demand): POST /api/sensors/batch — batch of up to 48 readings POST /api/ml/recommend — for fresh AI crop recommendation POST /api/v1/disease/detect — for cloud re-analysis of disease images Headers: Authorization: Bearer {device_jwt}, Content-Type: application/json Tier 3 → Tier 2 (response): AI recommendation → RPi → MQTT → ESP32 node (updates stored schedule) Cloud disease result → RPi → SMS/speaker/LCD alert to farmer

# Section 5: Smart Irrigation System

## 5.1  Irrigation Hardware Stack

| Component | Specification | Qty / Node | Cost (INR) | Purpose |
| --- | --- | --- | --- | --- |
| 4-Channel Relay Module | 5 V optocoupled, 10 A / 250 VAC per ch. | 1 | ₹120–180 | Control pump + 3 zone valves |
| 12 V Latching Solenoid | 3/4-inch BSP, normally closed, 50 ms pulse | 3–4 | ₹350–500 ea. | Zone valve — zero idle power draw |
| Submersible DC Pump | 12 V DC, 5 L/min, 3 m head | 1 | ₹600–900 | Drip irrigation for small farm |
| AC Centrifugal Pump | 0.5 HP 220 VAC, 30 L/min | 1 | ₹3,000–5,000 | Flood irrigation for large fields |
| Float Switch | NC type, 1 m cable | 1 | ₹80–120 | Pump dry-run protection |
| YF-S201 Flow Meter | Hall-effect, 1–30 L/min | 1 | ₹150–250 | Actual water volume monitoring |
| LM2596 Buck Converter | 3 A step-down 12 V → 5 V | 1 | ₹40–80 | Power ESP32 from 12 V pump supply |
| Manual Override Switch | SPDT toggle, 10 A rated | 1 per zone | ₹20–50 | Physical override for farmer |
| IRLZ44N MOSFET | Logic-level N-ch, 55 V, 47 A | 4 | ₹20–40 ea. | Drive latching valve H-bridge coils |
| 1N4007 Flyback Diode | 1 A, 1000 V rectifier | 8 | ₹2–5 ea. | Protect ESP32 from inductive kicks |

## 5.2  Wiring Description

ESP32 IRRIGATION CONTROLLER WIRING: Power rail: 12V SLA Battery (+) → 10A fuse → LM2596 (→ 5V → ESP32 VIN) → 12V rail (pump + solenoids) Relay module: ESP32 GPIO25 → Relay IN1 (COM/NC/NO) → Pump motor 12V+ ESP32 GPIO26 → Relay IN2 → Zone 1 solenoid coil (+) ESP32 GPIO27 → Relay IN3 → Zone 2 solenoid coil (+) ESP32 GPIO14 → Relay IN4 → Zone 3 solenoid coil (+) Relay VCC = 5V (from LM2596); JD-VCC = 5V (separate for isolation) All solenoid coil (-) → battery GND via shared bus Latching solenoid driver (per zone): ESP32 GPIO → IN+ of L298N half-bridge OR two MOSFETs in H-bridge Pulse HIGH (100 ms) to OPEN; Pulse reversed polarity to CLOSE Flyback diode (1N4007) across each solenoid coil — mandatory Safety interlocks: Float switch: GPIO35 (input pull-up) → GND. Low = tank empty → pump OFF Flow meter: GPIO34 interrupt → no flow after 5 min ON = leak/clog alert Manual override: GPIO32 (input pull-up) → SPDT switch. Physical switch overrides MQTT. All override events logged as NVS entry. ALL 220V AC WIRING MUST BE DONE BY LICENSED ELECTRICIAN. Use MCB: 2A for ESP32 circuit; 10A for AC pump circuit. Mount in separate locked AC compartment within enclosure (IP66).

## 5.3  AI Irrigation Decision Logic

IRRIGATION DECISION ENGINE (RPi Gateway + Cloud): Inputs: soil_moisture_pct, crop_type, growth_stage, temperature, humidity, rainfall_last_24h, ET0, tank_level_pct Rule 1 — Skip conditions (no irrigation today): soil_moisture > 65% → already sufficient rainfall_last_24h > 15 mm → natural irrigation sufficient tank_level_pct < 10% → refill alert + skip (pump protection) Rule 2 — Calculate irrigation amount (mm): deficit_mm = field_capacity_mm - (moisture_pct * effective_depth_cm * 10) ET0 = Hargreaves: 0.0023*(Tmean+17.8)*sqrt(Tmax-Tmin)*Ra gross_mm = (deficit_mm + ET0) / irrigation_efficiency efficiency: drip=0.90, sprinkler=0.75, flood=0.60 Rule 3 — Get AI crop confirmation: POST /api/ml/recommend with current N,P,K,temp,humidity,pH,rainfall If confidence>0.80: use crop-specific water requirement table duration_min = gross_mm / flow_rate_mm_per_min Rule 4 — Publish MQTT command: Topic: krishiai/{farm_id}/irrigation/command {valve_id:1, state:"on", duration_minutes:45, reason:"soil_deficit"} Rule 5 — Monitor and alert: Flow < expected after 5 min ON → ALERT: clogged dripper/broken pipe Soil moisture not rising after 30 min → ALERT: possible field leak Schedule 4 AM–6 AM (night irrigation to minimise evaporation)

# Section 6: Power System Design

## 6.1  Power Consumption Estimates — Field Node

| Component | Active Power | Sleep Power | Daily Wh (est.) |
| --- | --- | --- | --- |
| ESP32 (WiFi TX active) | 240 mA @ 3.3 V = 0.79 W | 10 µA deep sleep | ~0.10 Wh |
| DHT22 / BME280 | 1.5 mA @ 3.3 V = 5 mW | 0.1 mW | ~0.12 Wh |
| Capacitive soil sensor | 5 mA @ 3.3 V = 16 mW | 0 (GPIO-gated) | ~0.02 Wh |
| NPK RS485 sensor (polled) | 150 mA @ 12 V = 1.80 W | 0 (power-gated) | ~0.02 Wh |
| ESP32-CAM (capture burst) | 310 mA @ 3.3 V = 1.02 W | 2 µA deep sleep | ~0.12 Wh |
| LoRa SX1278 TX | 120 mA @ 3.3 V = 0.40 W | 0.2 mA standby | ~0.04 Wh |
| 4-ch Relay (1 active) | 70 mA @ 5 V = 0.35 W | 5 mA standby | ~0.35 Wh |
| 12 V Latching solenoid (pulse) | 1 A @ 12 V = 12 W (100 ms) | 0 W (latching) | ~0.001 Wh |
| TOTAL (mixed duty cycle) | ~3.5 W peak | ~50 mW sleep | ~0.77 Wh/day |

With 1.5× safety margin: 0.77 × 1.5 = 1.15 Wh/day = ~96 mAh/day at 12 V. A 12 V 7 Ah battery at 70 % DoD = 4,900 mAh → 51-day autonomy with zero solar charging. Practically, even 3 h/day of 20 W solar more than covers daily consumption.

## 6.2  Solar Panel Sizing

| Parameter | Field Node | Village Gateway |
| --- | --- | --- |
| Daily energy consumption | 1.15 Wh (with safety margin) | 240 Wh (RPi + modem + display + camera) |
| Worst-case sun hours/day | 4.0 h (monsoon, India average) | 4.0 h (monsoon) |
| System efficiency factor | 0.80 (dust, wire loss, PWM) | 0.85 (MPPT controller) |
| Theoretical panel needed | 1.15/(4.0×0.80) = 0.36 Wp | 240/(4.0×0.85) = 70.6 Wp |
| Recommended panel | 20 W (55× oversized for reliability) | 2× 40 W panels in parallel = 80 W |
| Why over-sized | 3× rule for monsoon/dust/shade | RPi peak load + future expansion |
| Panel dimensions | 485×360×17 mm (20 W) | 670×540×30 mm (40 W each) |

## 6.3  ESP32 Deep Sleep Strategy

FIRMWARE POWER OPTIMISATION — ESP32 DEEP SLEEP CYCLE: Normal 5-minute sensing cycle: 1. Wake from deep sleep (timer wakeup, ~10 ms boot) 2. GPIO26 → HIGH: enable P-MOSFET sensor power rail (DHT22 + BME280 + BH1750) 3. Wait 1 s for sensors to stabilise 4. Read all I2C + single-wire sensors in parallel (~500 ms) 5. GPIO26 → LOW: cut sensor power (saves 20 mA idle) 6. Check NVS: any irrigation command scheduled? → execute relay 7. Build JSON payload (N,P,K,pH,moisture,temp,rain,lux) 8. LoRa TX: send 200-byte packet to gateway (3 retries, 2 s timeout) 9. Store reading in SPIFFS circular buffer (72 h capacity) 10. Enter esp_deep_sleep_start() for 295 s (5 min minus ~5 s awake) Camera capture cycle (triggered externally): Wake source: GPIO39 external wakeup (farmer button press) Or: GPIO33 signal from RPi via transistor (MQTT-triggered capture) Capture JPEG → WiFi connect → HTTP POST to gateway → WiFi disconnect → sleep Average current = ~8 mA (vs 150–310 mA always-on). 20× power reduction.

## 6.4  Power System Summary Table

| System | Solar | Battery | Controller | Autonomy (no sun) | Total Cost |
| --- | --- | --- | --- | --- | --- |
| Field Node | 20 W Poly | 12 V 7 Ah SLA | PWM 10 A (₹150) | 51 days | ₹1,600–2,000 |
| Village Gateway | 2× 40 W Poly | 12 V 18 Ah SLA | MPPT 20 A (₹1,200) | 20 hours | ₹5,000–7,000 |
| Village Kiosk+AC | Grid + 40 W UPS | 150 Ah deep-cycle | Inverter 500 W | Grid-backed | ₹8,000–12,000 |

# Section 7: PCB and Circuit Design

## 7.1  Modular PCB Approach (3-Board System)

Instead of one monolithic PCB, KrishiAI uses three plug-together boards. If one board fails in the field, replace only that board — not the whole assembly. Each board can be independently prototyped on perfboard before PCB fabrication.

| Board | Name | Dimensions | Key ICs / Parts | Function |
| --- | --- | --- | --- | --- |
| Board A | KrishiAI Sensor Board | 100×80 mm | ESP32-WROOM-32, MAX485, PH4502C ADC, I2C header (×2), UART header | All sensor acquisition + local AI decisions |
| Board B | KrishiAI Power + Comms | 80×60 mm | SX1278 LoRa, SIM800L footprint, LM2596 buck, TP4056 slot, battery protection DW01A | Power management + wireless communications |
| Board C | KrishiAI Relay + Actuator | 80×60 mm | 4× PC817 optocoupler relay drivers, IRLZ44N MOSFETs, ACS712 current sense, screw terminals | Irrigation pump/valve control + electrical safety |

## 7.2  ESP32 GPIO Mapping

| GPIO | Function | Type | Connected To |
| --- | --- | --- | --- |
| GPIO 2 | Status LED | Output | Green LED + 470 Ω to GND — heartbeat blink codes |
| GPIO 4 | Sensor power rail gate | Output | P-MOSFET (IRLML2402) gate — cuts sensor 3.3 V rail in sleep |
| GPIO 12 | RS485 DE/RE | Output | MAX485 pin 2+3 — direction control for NPK MODBUS |
| GPIO 13 | LoRa NSS (CS) | Output (SPI) | SX1278 pin 13 — SPI chip-select |
| GPIO 14 | Relay 4 — Zone 3 valve | Output | 4-ch relay module IN4 (active-LOW logic) |
| GPIO 15 | LoRa RESET | Output | SX1278 pin 7 — hardware reset line |
| GPIO 16 | UART2 RX (NPK) | Input | MAX485 RO — receive from NPK sensor |
| GPIO 17 | UART2 TX (NPK) | Output | MAX485 DI — transmit MODBUS query |
| GPIO 18 | LoRa SCK | Output (SPI) | SX1278 SCK |
| GPIO 19 | LoRa MISO | Input (SPI) | SX1278 MISO |
| GPIO 21 | I2C SDA | Bidirectional | BME280 + BH1750 + DS3231 RTC |
| GPIO 22 | I2C SCL | Output | I2C clock — 400 kHz fast mode |
| GPIO 23 | LoRa MOSI | Output (SPI) | SX1278 MOSI |
| GPIO 25 | Relay 1 — Pump | Output | 4-ch relay IN1 |
| GPIO 26 | Relay 2 — Zone 1 valve | Output | 4-ch relay IN2 |
| GPIO 27 | Relay 3 — Zone 2 valve | Output | 4-ch relay IN3 |
| GPIO 32 | Manual override switch | Input pull-up | SPDT toggle switch → GND when closed |
| GPIO 33 | RPi wakeup / cam trigger | Input | RPi GPIO22 via 3.3 V transistor level shift |
| GPIO 34 | Flow meter pulse | Input interrupt | YF-S201 yellow wire (input-only GPIO) |
| GPIO 35 | Float switch | Input pull-up | NC float switch — HIGH = tank empty (input-only) |
| GPIO 36 (SVP) | Soil moisture ADC | ADC1 CH0 | Capacitive sensor AOUT |
| GPIO 39 (SVN) | Camera button wakeup | Ext. wakeup | Push button → GND (input-only, no pull-up) |

## 7.3  PCB Design Rules

| Parameter | Specification | Rationale |
| --- | --- | --- |
| PCB layers | 2-layer (prototype); 4-layer (V2) | Cost: 2-layer ₹500/5pcs at JLCPCB; 4-layer adds EMI shielding |
| Copper weight | 1 oz top + 1 oz bottom; 2 oz on power traces | 2 oz for 12 V / 2 A relay traces (1.5 mm minimum width) |
| Min trace width | 0.25 mm signal; 2.0 mm power | PCB house minimum 0.1 mm; 2 mm for 2 A current safety |
| Ground plane | Full copper pour on bottom layer | Reduces EMI from LoRa TX; improves RF ground for WiFi antenna |
| Antenna keep-out | 50 mm copper-free zone around ESP32 antenna | PCB copper near antenna can cut LoRa/WiFi range by 60 % |
| Decoupling caps | 100 nF ceramic per IC VCC + 10 µF bulk | Place within 3 mm of IC power pin — standard best practice |
| ESD protection | SP0503BAHTG TVS array on all I/O connectors | Field connectors routinely get static discharge |
| Surge protection | MOV varistor (275 V) on AC input + 1 A polyfuse on 12 V rail | Lightning induction via field cables is the #1 hardware killer |
| Connectors | 3.81 mm pitch screw terminals for field wiring; 2.54 mm headers inter-board | Screw terminals withstand vibration; headers for quick board swap |
| Conformal coating | MG Chemicals 422B acrylic after assembly | Monsoon moisture protection; apply 2 thin coats, mask USB/headers |
| PCB EDA tool | KiCad 7.0 (free, open-source) | Industry standard; direct JLCPCB / PCBWay Gerber export |
| Fab recommended | JLCPCB (5 pcs ₹500 + shipping) for prototype; Kaynes / SFO for production | JLCPCB: 7-day turnaround to India via DHL Express |

# Section 8: Bill of Materials (BOM)

## 8.1  MVP Low-Budget — 1 Field Node + Basic Gateway  (~₹13,500)

| Component | Model | Qty | Unit ₹ | Total ₹ | Source |
| --- | --- | --- | --- | --- | --- |
| ESP32 DevKit v1 | WROOM-32 38-pin | 1 | 300 | 300 | Amazon / Robu.in |
| ESP32-CAM | OV2640 2MP module | 1 | 320 | 320 | Amazon |
| Capacitive soil moisture | v1.2 corrosion-free | 2 | 100 | 200 | Robu.in |
| DHT22 temp+humidity | AM2302 with header | 1 | 150 | 150 | Amazon |
| pH sensor + board | E-201-C + PH4502C | 1 | 700 | 700 | Amazon |
| BME280 pressure+temp | GY-BME280 I2C breakout | 1 | 120 | 120 | Robu.in |
| BH1750 light sensor | GY-302 I2C breakout | 1 | 70 | 70 | Robu.in |
| SX1278 LoRa module | Ra-02 433 MHz (×2) | 2 | 250 | 500 | Robu.in |
| SIM800L GSM module | EVB with SIM holder | 1 | 380 | 380 | Amazon |
| 4-channel relay | 5V optocoupled | 1 | 150 | 150 | Amazon |
| 12V latching solenoid valve | 3/4-inch BSP NC | 2 | 450 | 900 | IndiaMART |
| MAX485 RS485 module | TTL ↔ RS485 breakout | 1 | 30 | 30 | Robu.in |
| LM2596 buck converter | 3A step-down 12V→5V | 2 | 60 | 120 | Amazon |
| 12V 7Ah SLA battery | Sealed Lead Acid | 1 | 550 | 550 | Amazon / local |
| 20W 12V solar panel | Polycrystalline | 1 | 750 | 750 | Amazon |
| PWM 10A charge controller | Generic CN3722 | 1 | 180 | 180 | Amazon |
| Raspberry Pi 4 2GB | RPi 4 Model B | 1 | 5200 | 5200 | Robu.in |
| 32GB MicroSD (RPi) | Samsung EVO Plus A1 | 1 | 450 | 450 | Amazon |
| IP66 junction box | 150×100×70 mm PC | 2 | 220 | 440 | Electrical shop |
| DS3231 RTC module | I2C real-time clock | 1 | 80 | 80 | Amazon |
| HC-SR04 ultrasonic | Water level sensor | 1 | 100 | 100 | Robu.in |
| Misc (wire, JST, screws) | 22AWG + connectors + PCB | 1 | 600 | 600 | Local |

MVP TOTAL: ~₹12,290  |  Add 10% buffer → ₹13,520

## 8.2  Mid-Range — 2 Field Nodes + Full Gateway  (~₹30,000)

| Addition Over MVP | Component | Qty | Cost ₹ |
| --- | --- | --- | --- |
| NPK RS485 professional sensor | RS-ECPHTH-N01 7-in-1 | 1 | 3,200 |
| Tipping bucket rain gauge | MISOL WH-SP-RG | 1 | 1,000 |
| Second complete field node | All sensors + solar + relay | 1 | 4,500 |
| RPi Camera Module v2 | IMX219 8MP CSI | 1 | 2,000 |
| 4G LTE USB modem | SIM7600G-H dongle | 1 | 2,200 |
| 3.5-inch TFT LCD (kiosk) | RPi GPIO touch display | 1 | 500 |
| Flow meter | YF-S201 hall-effect | 1 | 200 |
| MPPT 20A controller (upgrade) | Renogy Wanderer 20A | 1 | 1,100 |
| Speaker + amplifier | PAM8403 3W + 3W speaker | 1 | 200 |
| 12V 18Ah SLA (gateway) | For RPi gateway backup | 1 | 1,100 |
| 40W solar panel (gateway) | 2× 20W in parallel | 2 | 1,500 |

MID-RANGE TOTAL: ₹13,520 + ₹17,500 additions = ~₹31,000

## 8.3  Premium Scalable — 5 Nodes + Full AI Gateway  (~₹1,30,000)

| Item | Details | Cost ₹ |
| --- | --- | --- |
| 3 extra field nodes (full spec each) | ESP32 + NPK + pH + solar + relay × 3 | 42,000 |
| Atlas Scientific EZO-pH (lab-grade pH) | I2C pH × 3 (premium accuracy) | 15,000 |
| LoRaWAN 8-channel gateway hat | RAK2245 Pi HAT (replaces single SX1278) | 6,500 |
| Jetson Nano 4GB (edge AI upgrade) | NVIDIA Jetson Nano 4GB Developer Kit | 8,000 |
| LiFePO4 battery 12V 20Ah (gateway) | Long-life lithium iron phosphate | 6,000 |
| 7-inch HDMI display (permanent kiosk) | Waveshare 7-inch IPS + case | 2,000 |
| Waterproof CCTV dome housing (RPi cam) | IP66 vandal-proof dome | 700 |
| Custom PCB fabrication (5 complete sets) | JLCPCB 2-layer + hand assembly | 8,000 |
| Professional IP67 enclosures × 5 | Rittal or Spelsberg (industrial grade) | 6,000 |
| NDVI sensor add-on | Adafruit AS7265x Spectral | 3,500 |
| GPS module (each node) | NEO-6M I2C GPS for field mapping | 1,500 |

PREMIUM TOTAL: ₹31,000 + ₹99,200 = ~₹1,30,200 for full 5-node AI smart farm

## 8.4  Mass Production Cost Reduction Strategy

| Strategy | Savings % | How |
| --- | --- | --- |
| Custom SMD PCB (JLCPCB assembly) | 35–40% | Single PCBA replaces 8+ individual breakout modules |
| Bulk sensor procurement (50+ units) | 20–25% | Direct order from Shenzhen factory via Alibaba |
| ESP32-S3 (camera built-in) | 15% | ESP32-S3-CAM module = MCU + camera in single unit |
| Airtel/Jio M2M IoT SIM plan | 50% on data | ₹29–49/month M2M plan vs ₹150+ consumer SIM |
| Local PCB fab (Bangalore/Pune) | 10–15% | No customs + faster support for 500+ unit runs |
| LiFePO4 from Livguard/AMPERE India | 15% | India-made cells; no import duty |

# Section 9: Software-Hardware Integration

This is the most critical section — it shows EXACTLY how your ESP32 sensor hardware connects to the KrishiAI FastAPI backend that is already built and running.

## 9.1  How ESP32 Sensor Data Reaches /api/ml/recommend

ESP32 → /api/ml/recommend INTEGRATION YOUR EXISTING ENDPOINT accepts this exact JSON body: POST /api/ml/recommend { "N": 45, "P": 18, "K": 62, "temperature": 28.5, "humidity": 65.0, "ph": 6.8, "rainfall": 12.0 } YOUR HARDWARE PRODUCES all 7 values: N, P, K → RS-ECPHTH-N01 NPK sensor (MODBUS RS485) temperature → DHT22 or BME280 (or NPK sensor built-in temp) humidity → DHT22 or BME280 ph → E-201-C pH probe + PH4502C ADC board rainfall → Tipping-bucket rain gauge (pulse counter) ESP32 FIRMWARE (Arduino/PlatformIO) — pseudo-code: // 1. Read NPK sensor via MODBUS RS485 float N = npk.readRegister(0x0000); float P = npk.readRegister(0x0001); float K = npk.readRegister(0x0002); float ph = npk.readRegister(0x0006); // 2. Read DHT22 float temperature = dht.readTemperature(); float humidity = dht.readHumidity(); // 3. Rain gauge (interrupt counter, reset daily at midnight) float rainfall_mm = tipCount * 0.2; // 4. Build JSON payload String payload = "{"N":" + N + ","P":" + P + ","K":" + K + ","temperature":" + temperature + ","humidity":" + humidity + ","ph":" + ph + ","rainfall":" + rainfall_mm + "}"; // 5. POST to KrishiAI backend HTTPClient http; http.begin("https://your-api.onrender.com/api/ml/recommend"); http.addHeader("Content-Type", "application/json"); http.addHeader("Authorization", "Bearer " + DEVICE_JWT); int code = http.POST(payload); String response = http.getString(); // response: {"recommended_crop":"rice","confidence":0.87} // → store in NVS, display on LCD, announce via speaker

## 9.2  How ESP32-CAM Reaches /api/v1/disease/detect

ESP32-CAM → /api/v1/disease/detect INTEGRATION YOUR EXISTING ENDPOINT (scaffolded, needs CNN model wired in): POST /api/v1/disease/detect Content-Type: multipart/form-data Body: image file (JPEG) + crop_type + farm_id ESP32-CAM FIRMWARE — camera capture + upload: camera_config_t config; config.pixel_format = PIXFORMAT_JPEG; config.frame_size = FRAMESIZE_SVGA; // 800x600 config.jpeg_quality = 12; // 0=best, 63=worst esp_camera_init(&config); camera_fb_t *fb = esp_camera_capture(); // Upload as multipart/form-data WiFiClientSecure client; client.setInsecure(); // Use proper cert in production HTTPClient http; http.begin(client, "https://your-api.onrender.com/api/v1/disease/detect"); http.addHeader("Authorization", "Bearer " + DEVICE_JWT); // Build multipart body String boundary = "----KrishiAIBoundary"; // ... attach fb->buf (JPEG bytes) + crop_type field // Response: // {"detected_disease":"Rice Blast","confidence":0.91, // "remedy_suggested":"Tricyclazole 75 WP @ 0.6 g/L spray..."} WHAT TO ADD IN backend/api/ (disease endpoint stub): 1. Accept multipart/form-data with image + crop_type + farm_id 2. Save image to S3/local storage, get image_url 3. Run TFLite or call GPT-4o Vision with image_base64 4. INSERT INTO disease_detections (user_id, field_id, image_url, detected_disease, confidence, remedy_suggested) 5. Return JSON result to ESP32-CAM

## 9.3  Complete Sensor-to-API Data Flow Table

| Sensor on Hardware | Data Produced | Goes To | API Field Name | Stored In DB |
| --- | --- | --- | --- | --- |
| NPK RS485 sensor | Nitrogen mg/kg | /api/ml/recommend | "N" | farmer_fields.soil_type (derived) |
| NPK RS485 sensor | Phosphorus mg/kg | /api/ml/recommend | "P" | farmer_fields (future npk_json) |
| NPK RS485 sensor | Potassium mg/kg | /api/ml/recommend | "K" | farmer_fields (future npk_json) |
| DHT22 / BME280 | Temperature °C | /api/ml/recommend | "temperature" | sensor_readings table (new) |
| DHT22 / BME280 | Humidity % | /api/ml/recommend | "humidity" | sensor_readings table (new) |
| pH probe + PH4502C | Soil pH (0–14) | /api/ml/recommend | "ph" | sensor_readings table (new) |
| Tipping rain gauge | Rainfall mm/24h | /api/ml/recommend | "rainfall" | sensor_readings table (new) |
| ESP32-CAM (OV2640) | Leaf JPEG image | /api/v1/disease/detect | image file | disease_detections.image_url |
| Capacitive moisture | Soil moisture % | /api/sensors/batch | moisture | sensor_readings table (new) |
| BME280 | Pressure hPa | /api/weather/health | (local only) | sensor_readings table (new) |
| BH1750 | Light lux | /api/sensors/batch | lux | sensor_readings table (new) |
| HC-SR04 | Tank water level % | /api/sensors/batch | tank_pct | irrigation_events table (new) |
| YF-S201 flow meter | Water flow L/min | /api/sensors/batch | flow_lpm | irrigation_events table (new) |
| GPS module (NEO-6M) | Lat / Lon | /api/devices/register | lat, lon | users.lat, users.lon |

## 9.4  New Database Tables to Add

Add these tables to backend/db/models.py: class SensorReading(Base): __tablename__ = "sensor_readings" id = Column(Integer, primary_key=True) farm_id = Column(String, ForeignKey("farmer_fields.id")) N, P, K = Column(Float), Column(Float), Column(Float) pH = Column(Float) moisture = Column(Float) # % volumetric temperature = Column(Float) # °C humidity = Column(Float) # % RH rainfall = Column(Float) # mm/24h lux = Column(Float) # ambient light pressure = Column(Float) # hPa recorded_at = Column(DateTime, default=datetime.utcnow) class IrrigationEvent(Base): __tablename__ = "irrigation_events" id = Column(Integer, primary_key=True) farm_id = Column(String, ForeignKey("farmer_fields.id")) zone_id = Column(Integer) # 1, 2, 3 trigger = Column(String) # "ai_auto", "manual", "schedule" duration_min = Column(Integer) volume_litres= Column(Float) tank_pct = Column(Float) started_at = Column(DateTime) ended_at = Column(DateTime)

# Section 10: Firmware Architecture

## 10.1  ESP32 Firmware Folder Structure (PlatformIO)

ESP32 FIRMWARE STRUCTURE krishiai-firmware/ ├── platformio.ini # board: esp32dev; framework: arduino ├── src/ │ ├── main.cpp # setup() + loop() — orchestrates all modules │ ├── config.h # WiFi SSID, MQTT broker IP, API URL, device JWT │ ├── sensors/ │ │ ├── npk_sensor.cpp # MODBUS RS485 via HardwareSerial2 + MAX485 │ │ ├── ph_sensor.cpp # ADC read + 2-point calibration (NVS stored) │ │ ├── soil_moisture.cpp # Capacitive ADC + dry/wet calibration │ │ ├── dht22.cpp # Single-wire DHT22 + SHT31 I2C fallback │ │ ├── bme280.cpp # I2C BME280 pressure/temp/humidity │ │ ├── bh1750.cpp # I2C lux sensor │ │ ├── rain_gauge.cpp # Interrupt counter (GPIO39); resets at midnight │ │ └── water_level.cpp # HC-SR04 ultrasonic distance → tank % │ ├── comms/ │ │ ├── wifi_mqtt.cpp # WiFi connect + PubSubClient MQTT publish/subscribe │ │ ├── lora_comms.cpp # LoRa SX1278 send/receive via SPI │ │ ├── http_api.cpp # HTTPClient POST to /api/ml/recommend + disease/detect │ │ ├── gsm_sms.cpp # SIM800L AT commands — SMS send + receive │ │ └── ota_update.cpp # esp_https_ota — check manifest + apply update │ ├── irrigation/ │ │ ├── relay_control.cpp # GPIO relay + MOSFET latching solenoid driver │ │ ├── flow_meter.cpp # Pulse interrupt → L/min + total volume │ │ └── schedule.cpp # Stored schedule from NVS; offline fallback logic │ ├── storage/ │ │ ├── nvs_config.cpp # Preferences NVS: device ID, calibration, schedule │ │ └── spiffs_buffer.cpp # Circular JSON buffer — 72 h offline storage │ └── power/ │ └── sleep_manager.cpp # Deep sleep + wakeup sources management ├── test/ │ └── test_sensors.cpp # Unity unit tests for each sensor driver └── data/ └── config.json # Runtime config overlay on SPIFFS

## 10.2  RPi Gateway Services

RPi GATEWAY SERVICE STRUCTURE krishiai-gateway/ ├── requirements.txt # paho-mqtt, tflite-runtime, flask, httpx, pyserial ├── services/ │ ├── mqtt_bridge.py # Mosquitto subscriber → routes to handlers │ ├── tflite_inference.py # Load model → preprocess 224x224 → infer → return │ ├── cloud_sync.py # POST /api/sensors/batch every 15 min │ ├── sms_handler.py # pyserial AT commands to SIM800L for SMS relay │ ├── lora_gateway.py # Receive LoRa packets from out-of-WiFi nodes │ ├── knowledge_cache.py # SQLite: mandi prices + schemes + remedies offline │ └── ota_server.py # Serve firmware.bin to ESP32 nodes on LAN ├── models/ │ ├── plantvillage_mbv2_int8.tflite # Disease detection model (~5 MB) │ └── crop_rf_model.pkl # scikit-learn crop recommendation ├── db/ │ └── gateway.db # SQLite: sensor_buffer, disease_log, config ├── dashboard/ │ └── app.py # Flask on :8080 — local WiFi kiosk dashboard └── systemd/ ├── krishiai-gateway.service # Auto-start on RPi boot └── mosquitto.conf # MQTT broker: auth per device, TLS port 8883

## 10.3  MQTT Topic Summary and Frameworks

| Framework / Library | Platform | Version | Purpose |
| --- | --- | --- | --- |
| PlatformIO + Arduino | ESP32 | Latest | Dev environment — better library manager than IDF alone |
| PubSubClient | ESP32 | 2.8.0 | Lightweight MQTT client; QoS 0/1; widely used in India IoT |
| LoRa (Sandeep Mistry) | ESP32 | 0.8.0 | Clean SX1278 SPI driver; interrupt-driven RX; proven reliable |
| Preferences.h (NVS) | ESP32 | built-in | Atomic key-value store in flash; survives power loss |
| esp_https_ota | ESP32 | built-in | Secure OTA firmware update; SHA-256 hash verification |
| paho-mqtt | RPi Python | 1.6.1 | Eclipse Paho client; async callbacks; wildcard subscriptions |
| tflite-runtime | RPi Python | 2.14 | Lightweight TFLite without full TensorFlow (200 MB vs 2 GB) |
| pyserial | RPi Python | 3.5 | AT command interface to SIM800L via /dev/ttyS0 |
| Flask | RPi Python | 3.0 | Local dashboard on port 8080 for village kiosk WiFi access |
| Mosquitto | RPi Linux | 2.0 | MQTT broker; Debian package; low RAM; per-device ACL |
| httpx (async) | RPi Python | 0.27 | Same library as FastAPI backend — consistent HTTP client |

# Section 11: Edge AI and Cloud AI Architecture

## 11.1  AI Workload Distribution

| AI Task | Edge (RPi) | Cloud (FastAPI) | Reason |
| --- | --- | --- | --- |
| Crop disease detection | YES — TFLite INT8 MobileNetV2 (<200 ms, offline) | YES — GPT-4o Vision / CNN (higher accuracy + Hindi remedy) | Edge: instant offline result. Cloud: detailed remedy from disease_diagnosis_protocol.md knowledge. |
| Crop recommendation | YES — sklearn .pkl cached model | YES — /api/ml/recommend (latest trained weights) | Edge uses cached model; cloud has the freshest weights. Both produce same JSON schema. |
| Irrigation scheduling | YES — ET0 formula + rules (no internet needed) | YES — weather forecast + AI-enhanced schedule | Edge guarantees automation. Cloud adds 7-day weather forecast for proactive scheduling. |
| Voice chatbot STT | Optional — whisper.cpp tiny (3× slower, needs RPi 5) | YES — Whisper-1 API (/api/chat/stt endpoint) | Whisper-1 is cloud-only by default in your codebase. whisper.cpp is V2 offline option. |
| Multilingual LLM chat | NO — GPT-4o too large | YES — GPT-4o-mini / Gemini (/api/chat/message SSE) | LLM needs 8 GB+ RAM. RPi 4 2 GB cannot run it. Cloud-only for MVP. |
| Language detection | YES — regex (Devanagari/Kannada Unicode ranges) | YES — langdetect fallback in language_detect.py | Script detection is near-zero cost and already in your codebase for ASCII Latin text. |
| Mandi price prediction (V2) | NO | YES — LSTM time-series on e-NAM historical data | Requires large historical dataset. Cloud-only; display cached results on edge. |
| NDVI crop health (V2) | YES — simple NDVI formula from AS7265x sensor | YES — satellite NDVI from Sentinel-2 API | Edge: real-time single-point from NDVI sensor. Cloud: field-wide spatial map. |

## 11.2  TFLite Model Build Pipeline

MODEL TRAINING & OPTIMISATION PIPELINE: Step 1 — Train on Google Colab (free T4 GPU, ~2 hours): import tensorflow as tf base = tf.keras.applications.MobileNetV2(input_shape=(224,224,3), include_top=False, weights="imagenet") base.trainable = False # freeze backbone model = tf.keras.Sequential([base, tf.keras.layers.GlobalAveragePooling2D(), tf.keras.layers.Dense(38, activation="softmax")]) model.compile(optimizer="adam", loss="categorical_crossentropy") model.fit(train_ds, epochs=20, validation_data=val_ds) model.save("plant_disease_model.h5") Step 2 — Convert to TFLite (INT8 quantization): converter = tf.lite.TFLiteConverter.from_keras_model(model) converter.optimizations = [tf.lite.Optimize.DEFAULT] converter.representative_dataset = representative_data_gen converter.target_spec.supported_ops = [tf.lite.OpsSet.TFLITE_BUILTINS_INT8] converter.inference_input_type = tf.int8 converter.inference_output_type = tf.int8 tflite_model = converter.convert() open("plantvillage_int8.tflite","wb").write(tflite_model) # Result: ~5 MB model, ~180 ms on RPi 4 (all 4 cores) Step 3 — Deploy to RPi Gateway: scp plantvillage_int8.tflite pi@gateway-ip:/home/pi/krishiai-gateway/models/ # OR: OTA download when gateway has internet Step 4 — RPi inference call (services/tflite_inference.py): interpreter = tf.lite.Interpreter("models/plantvillage_int8.tflite") interpreter.set_num_threads(4) # use all RPi cores interpreter.allocate_tensors() # Preprocess: cv2.resize(img,(224,224)) / 255.0 # Run: interpreter.invoke() # Output: top-3 class indices + confidence scores

# Section 12: Security Architecture

## 12.1  Security Layers

| Layer | Threat | Solution | Implementation |
| --- | --- | --- | --- |
| Device Auth | Rogue device injecting fake soil data | Pre-provisioned device JWT + HMAC-SHA256 message signing | JWT stored in ESP32 NVS with flash encryption. Rotated via OTA monthly. |
| API Transport | Man-in-the-middle attack on sensor data | TLS 1.3 for all HTTPS. Certificate pinning on RPi gateway. | ESP32 WiFiClientSecure + root CA cert. httpx on RPi with verify=True. |
| MQTT Broker | Unauthorised commands to irrigation relay | Mosquitto with per-device username+password + ACL topic rules | Each ESP32 has unique client_id. ACL: node can only publish its own farm topics. |
| LoRa Packets | RF spoofing / replay of irrigation commands | AES-128-CTR payload encryption + 4-byte rolling counter (anti-replay) | mbedtls AES on ESP32 firmware. Counter stored in NVS; cloud validates. |
| Farmer Privacy | GPS + yield data leak | farm_id is UUID — never the phone number. No PII in sensor payloads. | Users table has phone; sensors table has farm_id only. Decoupled. |
| Cloud API | DDoS / brute-force on FastAPI | slowapi 60 req/min already in main.py. Add Cloudflare WAF in production. | Already implemented. Add Cloudflare Turnstile on PWA login for bot protection. |
| OTA Firmware | Malicious firmware pushed to field devices | Ed25519 signed firmware manifest + SHA-256 hash before applying | Sign firmware.bin at build time. ESP32 verifies signature; rolls back on failure. |
| Physical | PCB reverse engineering / key extraction | ESP32 flash encryption + secure boot burned in factory (efuse) | idf.py efuse-related commands. One-time burn at provisioning. Cannot be undone. |

# Section 13: Field Deployment Strategy

## 13.1  Village-Level Deployment Model

1 VILLAGE GATEWAY serves 50-100 farmers within 5 km radius. 1 FIELD NODE covers 2 acres (install more for larger farms). PHASE 1 — KVK Partnership (Month 1–3): Partner with local Krishi Vigyan Kendra (KVK) or gram panchayat. Install gateway at KVK office (permanent power + internet available). Install 3–5 demo field nodes on pilot farmer fields. Train local "KrishiAI Sakhi" (extension worker) as system operator. PHASE 2 — Early Adopters (Month 4–6): 20–30 farmers within 5 km of gateway. ₹500 refundable deposit + ₹99/month subscription. KrishiAI owns hardware; farmer pays service fee. PHASE 3 — Scale (Month 7–18): 50 villages; 1 field engineer per 20 villages. Remote monitoring dashboard for all gateways via cloud. OTA updates pushed centrally; no site visit needed.

## 13.2  Physical Installation and Weatherproofing

| Challenge | Solution | Specification |
| --- | --- | --- |
| Monsoon rain (1500–3000 mm/yr) | IP66 enclosure + M16 neoprene cable glands | ABS/PC box, IP66 minimum; replace gland seals annually |
| Summer heat (45°C+) | Polycarbonate enclosure + GoreTex vent membrane | PC rated to 120°C; vent prevents condensation; max ESP32 junction 85°C |
| Dust (dry Deccan season) | IP6X dust rating + filtered vent on box | Annual maintenance: blow out dust with compressed air |
| Condensation / humidity | Silica gel pack inside + conformal coat on PCB | Replace silica gel packet annually; MG Chemicals 422B on PCB |
| Rodent cable damage | Stainless-steel corrugated conduit for underground runs | 20 mm SS conduit; pepper powder at cable entry to soil |
| Lightning / surge | Lightning rod on pole + MOV on 12 V input + MCB | Copper ground rod 1.5 m into soil at pole base |
| UV degradation | UV-stabilised PC enclosure + UV-rated outdoor cable | Ensure UL 508A or IP66 UV-grade certification on enclosure label |
| Theft | Padlock + security Torx screws + GPS tracker in gateway | M5 Torx pin-in-head screws; GPS module hidden inside RPi enclosure |

## 13.3  Pole Installation Guide

STANDARD KRISHIAI FIELD NODE POLE INSTALLATION: Materials: 2-inch GI pipe 3 m length (₹350), concrete PCC 1:3:6, 4× M6 SS bolts, cable conduit 20 mm corrugated SS Steps: 1. Dig 60 cm deep hole, 20 cm diameter 2. Set GI pipe vertical (use spirit level), pour PCC concrete 3. Allow 24 h curing before mounting enclosure 4. Solar panel bracket at top: 45° tilt, facing True South 5. Enclosure at 2.0 m height (above flood level + reach) 6. ESP32-CAM bracket at 2.2 m, angled 30° downward for leaf shots 7. Rain gauge at 1.5 m, levelled with bubble level 8. Soil probes: insert 15 cm deep in representative soil 1 m from pole 9. All cables in conduit from pole base to soil sensors 10. Ground rod: 1.5 m copper rod driven at pole base, connected to enclosure earth Time per installation: ~4 hours, 2-person team Tools needed: shovel, spirit level, drill (cordless), crimping tool, multimeter Maintenance cycle: monthly visual check; quarterly sensor calibration; annual full service

# Section 14: Future Scalability — V2 and V3 Roadmap

## 14.1  V2 Hardware Additions (6–12 Months Post-MVP)

| V2 Feature | Technology | Benefit | Cost Add |
| --- | --- | --- | --- |
| NDVI / spectral health sensor | Adafruit AS7265x 18-channel spectral | Quantify chlorophyll, photosynthesis, plant stress without waiting for disease symptoms | ₹2,000–3,000 |
| CO2 / gas sensor | MH-Z19B NDIR CO2 (I2C) | Detect grain fermentation, greenhouse CO2 for plant growth optimisation | ₹800–1,200 |
| LoRaWAN 8-ch concentrator | RAK2245 Pi HAT (replaces single SX1278) | Scale from 1 to 100+ nodes on single gateway; true LoRaWAN Class A | ₹6,500 GW upgrade |
| NB-IoT fallback | Quectel BC660K-GL module | Penetrates buildings + basements; 20 dB better than 4G in deep rural | ₹600–900 per node |
| Offline LLM chatbot | Ollama + Llama 3.2 1B Q4 on RPi 5 8GB | Fully offline Hindi/Kannada chatbot — no internet needed at all | ₹8,000 RPi 5 upgrade |
| ORP sensor (water quality) | Atlas Scientific EZO-ORP | Monitor irrigation water redox potential — detects chemical contamination | ₹4,500 per node |
| Leaf wetness sensor | LWS-L or DIY PCB trace sensor | Predict fungal disease risk from leaf wetness duration | ₹200–500 |

## 14.2  V3 Hardware — Smart Farm Automation (12–24 Months)

| V3 Feature | Technology | Scale / Impact |
| --- | --- | --- |
| Autonomous spraying drone | Custom hexacopter + GPS + precision nozzles + ROS2 | 50-acre targeted spray in 30 min; 90 % pesticide reduction vs manual |
| Computer vision towers | 4K camera on 8 m tower + NVIDIA Jetson Orin NX | 360° field monitoring, real-time YOLO pest/disease heatmap |
| Smart greenhouse | CO2 + EC + humidity + shade actuators + fans | Controlled environment for high-value crops (strawberry, capsicum) |
| Satellite NDVI integration | Sentinel-2 API (free) + ISRO Bhuvan API | Weekly crop stress map overlaid on farmer field GeoJSON polygon |
| AI pest prediction | LSTM on historical weather + outbreak DB | Predict pest arrival 7–14 days in advance; proactive pesticide alerts |
| Village solar microgrid | 10 kW solar + 50 kWh battery + smart inverter | Power KrishiAI infra + village homes; cross-subsidise service cost |
| Autonomous field robot | RPi + ROS2 + tracked platform + soil probe arm | Auto-navigate between rows; spot-sample soil every 5 m |

# Section 15: MVP Build Plan and Scaling Roadmap

## 15.1  4-Week Prototype Build Plan

| Week | Tasks | Deliverable / Success Criteria |
| --- | --- | --- |
| Week 1 | Order all components. Setup PlatformIO. Blink LED + serial print on ESP32. | All parts received. Dev environment working. Hello World on serial monitor. |
| Week 2 | Wire and test each sensor individually on breadboard. Validate readings against known references (e.g., known-pH buffer solution). | Every sensor producing correct values. Calibration coefficients noted in spreadsheet. |
| Week 3 | Integrate all sensors on single ESP32. MQTT to local Mosquitto. Test deep-sleep cycle. Wire relay + solenoid valve on bench. | ESP32 publishing all 7 sensor values to MQTT every 5 min. Valve opens/closes on MQTT command. |
| Week 4 | RPi gateway: Mosquitto + Python MQTT bridge + TFLite model. Test disease detection on 20 leaf photos. Wire ESP32-CAM. | Full end-to-end: sensor → MQTT → RPi → POST /api/ml/recommend → crop name on serial. Camera capture → disease name returned. |

## 15.2  Pilot and Scale Milestones

| Phase | Timeline | Target | Investment | Revenue Model |
| --- | --- | --- | --- | --- |
| Prototype | Month 1 | 1 gateway + 3 nodes, 1 pilot village | ₹50,000 | None (R&D) |
| Pilot | Month 2–3 | 30 farmers, 3 villages, 2 districts | ₹2,50,000 | ₹99/month × 30 = ₹3,000/mo |
| Early Scale | Month 4–9 | 500 farmers, 50 villages | ₹25,00,000 | ₹99/mo × 500 = ₹49,500/mo |
| Series A | Month 10–24 | 10,000 farmers, 5 states | ₹2,00,00,000 | ₹99/mo × 10K = ₹9.9 L/mo |
| National | Year 3+ | 1,00,000 farmers, pan-India | Series B + Govt | PM-KISAN portal integration |

# Section 16: CTO-Level Recommendations and Final Summary

## 16.1  Top 10 Hardware Decisions for Indian Market

| Priority | Decision | Rationale |
| --- | --- | --- |
| 1 | Capacitive soil sensor — NOT resistive | Resistive sensors corrode in 1–2 monsoon cycles. Capacitive last 3–5 years. Never compromise on this. |
| 2 | 12 V SLA battery — NOT Li-Ion for field | SLA handles 0–50°C field temps without BMS. Available at every Indian town. Li-Ion needs temperature management. |
| 3 | LoRa for sensor network — NOT WiFi mesh | WiFi range in open fields = 50–100 m. LoRa = 3–5 km. One gateway covers entire village. |
| 4 | SMS fallback is non-negotiable | 55 % of Indian farmers use feature phones. SMS delivers crop alerts even with no smartphone. |
| 5 | Deploy real NPK sensor (RS485 7-in-1) | NPK is the core input to /api/ml/recommend. Mock or single-axis sensors give unreliable recommendations. |
| 6 | Oversize solar 3× | Monsoon clouds + dust + bird droppings reduce output. 3× oversizing = reliable power year-round. |
| 7 | IP66 enclosures from Day 1 | India's monsoon + 45°C summer will destroy unprotected electronics in one season. Never skip IP66. |
| 8 | Partner with KVK for credibility | Farmers trust the government extension system. KVK endorsement accelerates adoption 10×. |
| 9 | Hindi language above all others for MVP | 65 % of target farmers are Hindi-speaking. Kannada/Tamil/Marathi support can come in V2. |
| 10 | Design for 2G + LoRa first; WiFi second | Assume zero WiFi in the field. Every pipeline must work on 2G + LoRa only. |

## 16.2  Final Architecture Summary

FINAL HARDWARE SUMMARY KRISHIAI COMPLETE HARDWARE ECOSYSTEM — SUMMARY FIELD NODE (per 2 acres): ₹8,000–12,000 ESP32 DevKit + RS-ECPHTH-N01 NPK+pH 7-in-1 + DHT22 + BME280 + BH1750 ESP32-CAM (OV2640) + Tipping rain gauge + SX1278 LoRa 4-ch optocoupled relay + 12V latching solenoids (3 zones) 12V 7Ah SLA battery + 20W solar panel + PWM 10A controller IP66 polycarbonate enclosure on 3 m GI pole VILLAGE GATEWAY (per 50-100 farmers): ₹15,000–20,000 RPi 4 2GB + RPi Camera v2 + SX1278 LoRa RX + SIM7600 4G USB dongle SIM800L SMS + 3.5" TFT LCD + PAM8403 speaker + microphone TFLite PlantVillage disease model + Mosquitto MQTT broker 40W solar (×2) + 12V 18Ah SLA + MPPT 20A controller IP66 weatherproof enclosure at KVK / panchayat office CLOUD BACKEND (existing KrishiAI software — no changes needed for MVP): FastAPI + PostgreSQL + Redis on Render / AWS Mumbai ap-south-1 POST /api/ml/recommend ← ESP32 sends N,P,K,temp,humidity,pH,rainfall POST /api/v1/disease/detect ← ESP32-CAM sends JPEG leaf image POST /api/chat/message ← Farmer voice → RPi mic → Whisper → GPT-4o GET /api/mandi/crops ← Gateway caches every 6h for offline LCD display MVP TOTAL (1 gateway + 3 field nodes + installation): ₹45,000–56,000 BUSINESS MODEL: ₹499/month SaaS per farmer + ₹1,000 hardware deposit BREAK-EVEN PER FARMER: 8–10 months after installation

— End of KrishiAI Hardware + IoT + Edge AI Architecture Document —

Version 1.0  |  May 2026  |  KrishiAI Startup  |  CONFIDENTIAL